# URL Considerations for DTV Environments

Gomer Thomas

LGE Research Center of America, Inc.

40 Washington Road

Princeton Junction, NJ 08550

Phone: 609-716-3513

Fax: 609-716-3503

Email: gomer@lgerca.com

This note attempts to provide some background information on digital TV (DTV) environments, and then some requirements for URL schemes for DTV. If you are very familiar with the DTV standards, skip the background.

### BACKGROUND:

A DTV receiver may have no connection with the outside world through any link other than a DTV broadcast signal coming from an antenna, satellite, or cable feed. It may or may not have access to the Internet. It may or may not have access to devices on a home network (which may not be an IP network). For the moment I wil ignore the home network issues.

The signal coming from an antenna, satellite, or cable feed will typically have a number of RF bands on which DTV signals can be received. When the receiver tunes to a particular RF band and demodulates the signal there, it ends up with an MPEG-2 transport stream. There are currently two competing standards for DTV, the DVB standard developed in Europe, and the ATSC standard developed in the United States. Among other things, they use different methods for modulating the digital bit stream into the RF signal, so in the near term at least most receivers will be able to receive one or the other, but not both.

I am much more familiar with the ATSC standard than the DVB standard, so the following explanations are oriented toward the ATSC standards. Much of it, but not all, will be true for DVB as well.

Each MPEG-2 transport stream has a transport_stream_id. There is currently no guarantee of uniqueness of these IDs in the ATSC world. However, the National Assn of Broadcasters (NAB) is advocating that the FCC assign transport stream IDs to broadcasters, associated with their broadcast license, so that they would be guaranteed to be unique. In the meantime, they may well be unique in practice.

Each MPEG-2 transport stream may contain multiple "programs" or virtual channels. (The ATSC DTV standards use the term "event" to describe what would be called a TV program in normal usage, and they use the term "program" to describe what would be called a TV channel in normal usage.) For example, a single transport stream may contain one high definition TV (HDTV) program, or several standard definition TV (SDTV) programs, along with a potentially large number of pure data programs. Each virtual channel has associated with it a program_number, a major and minor channel number, and a source_id. All of    these are guaranteed to be unique within the transport stream, but not across transport streams. Each of these may also have associated with it a short channel name. The number of programs in a single transport stream may be as few as one, or as many as dozens, depending on the bandwidth required for the programs. The number may vary over time.

Each program may contain multiple elementary streams. For example, a typical TV program would contain one video stream and perhaps multiple audio streams in different languages. An enhanced TV program might also contain multiple streams containing data to support interactive applications to enhance the viewing of the program -- for example, statistics on demand during a baseball game, or biographical information on demand during a talk show. There might be pure data programs which contain only one or more elementary streams of data to support some application, for example a stock ticker or news headline ticker which runs across the bottom of the screen, independently of what video program is being watched. Each elementary stream has a packet_id and optionally an association_tag. These are guaranteed to be unique within the program.

A question related to uniqueness is stability. If I tune to the same RF frequency from the same broadcast station, will the resulting transport stream always have the same ID? In terms of guarantees, no, but in practice probably yes. If I look at channel 17-2 (major-minor channel number) from the same station, will it always have the same program_number and source_id? In terms of guarantees, no in both cases. In practice, probably no for the program_number and yes for the source_id. If I look at the video for channel 17-2, will it always have the same packet_id? Almost certainly not.

Another form of stability is what happens during the broadcasting process. Typically the multiple programs which may appear in a transport stream are assembled in the broadcast facility more or less independently and are then multiplexed together into the final transport stream. What happens to the various identifiers during this multiplexing process? The program_number and elementary stream packet_ids are likely to change. The others will not.

Note also that the same network TV program, say from PBS, could be broadcast by numerous local affiliate stations, and the RF band, transport stream ID, major-minor channel number, and channel name would be different for each affiliate. Thus, receivers in different parts of the country would find the same program (with exactly the same content) under different RF bands, transport stream IDs, channel numbers, and channel names. It might also be broadcast at different times -- not only different local times, but different absolute times.

To complicate things further, when a program is picked up by a cable or satellite operator and rebroadcast, nearly every single one of the identifiers for the program may be changed -- RF frequency, ts_id, program_number, major-minor channel number, source_id, packet ids, possibly even association tags. Proably the channel name will stay the same.

Each TV program (or "event") has an event_id. However, currently these event_ids are only guaranteed to be unique within the set of events in the same transport stream which appear in the same 3-hour interval (where each day is divided into eight standard 3-hour intervals, measured in GMT). A proposal has been submitted to ATSC to make the event_ids unique within each source_id, but this is being resisted by the broadcasters so far, due to certain complexities it would introduce into their operations.

The ATSC Data Broadcast Specification (from T3/S13 subcommittee) specifies a number of different protocols for including data in an MPEG-2 broadcast stream:

- DSM-CC bounded data carousels

- DSM-CC unbounded data carousels

- streaming data in PES packets

- IP or other protocol datagrams encapsulated in PES or TS packets

- piped data in TS packets

There is a strong probability that at some point in the future either the T3/S13 (Data Broadcast) or T3/S16 (Interactive Applications) subcommittee will specify the use of DSM-CC object carousels as well, but that does not appear likely to happen right away.

In most cases this data can be:

- asynchronous: presentation timing more or less arbitrary

- synchronized: presentation timing tied to other streams, such as audio/video

- synchronous: presentation timing internally constrained

Even data which is essentially stable will usually be broadcast repeatedly at fairly short intervals, to accommodate viewers who tune into the middle of an event, or receivers which have little or no caching capability.

The ATSC Data Broadcast Specification also specifies a Service Description Table which is used to describe the applications which make up a data event, and specify the files the applications need and where to find them. Thus, within a given event, an application_name and so-called tag_id can be used to identify a data resource (file).

### REQUIREMENTS:

A number of broadcasters will be broadcasting data enhancements along with their video events, and even broadcasting pure data events, of the sort described earlier in this note. In many cases the data will be in the form of HTML pages, to be rendered onto the TV screen by HTML engines in the receivers. Although we often think of an HTML page as a single file, it usually requires a number of files to render even a single page, just for the images which appear in the page if nothing else. Moreover, an application may often contain multiple inter-linked HTML pages. All of these files will have to be included in the broadcast stream, and the HTML engine will have to be able to find the files from URLs which appear in the HTML pages.

To complicate things, the HTML pages for the application will often be generated by some content creator far removed from the eventual broadcast stations. It will then be sent to the network, which will distribute it around to the different local affiliates to include in their broadcast. Thus, all of the various identifiers described in the background section of this note will be unknown to the content creator.

The simplest case is that all of the inter-linked files appear in the same elementary stream. The next simplest case is that they all appear in the same program, but possibly in different elementary streams.

Since a single transport stream may include four or so SDTV programs, it is likely that at times there will be some coordination of data applications across multiple virtual channels within the same transport stream, so a URL which appears in one program may reference data which appears in another program.

Moreover, in the cable world there may often be a single broadcaster controlling multiple transport streams. In fact, with the increasing concentration in the communications industry, this may happen in the not too distant future for terrestrial broadcasts as well. Thus, there may be coordinated data applications across multiple transport streams, so a URL which appears in one program may reference data which appears in other transport streams.

Then, of course, some DTV sets will have Internet access (and some PCs will have TV access).

Thus, an HTML page contained in a broadcast may reference files in the Internet, to give TV viewers access to additional information beyond what is included in the DTV broadcast, or to give TV viewers the ability to interact with web servers to order products, etc.

Also, broadcasters and others will have web sites, where they will want to reference broadcast programs. In some cases they may want to reference broadcast video eventss to watch. In other cases they may want to reference broadcast data events, or individual files which are included in a broadcast data event. For example, a stockbroker who wants to advertise a live TV stock ticker data service might well provide a reference to it on his/her web site. In the case of a TV event which is only available at certain times, the URL must include a time component of some sort.

All this leads to the following requirements for URL schemes for DTV environments:

- URLs must actually be URLs, not just URIs. Given a URL, it must be possible for a receiver to actually locate the resource, or conclude that it is not reachable.

- It must be possible for the resource referenced by a URL to be an entire program/event, or just a single component of a program/event.

- It must be possible for a URL to contain an identification of the date(s) and time(s) when the resource is available, either through an explicit time designation, or through an event designation.

- The set of URL schemes for DTV should include audio/video streams and all of the data broadcast protocols endorsed by the ATSC Data Broadcast Specification.

- A URL must be meaningful when interpreted from any of the following locations relative to the resource being referenced:

- From within the same elementary stream

- From within the same program

- From within the same transport stream

- From within a different transport stream received by the same receiver

- From an arbitrary location in the Internet

The correct interpretation of this requirement is that a DTV receiver should be able to obtain an HTML page from any of the listed locations, and if the resource referenced by a DTV URL in that page is available to that receiver in the broadcast streams it can receive, it should be able to find the resource.

Note that locating the resource may require intermediate steps, analogous to a DNS lookup on the Internet. ATSC T3/S8 document 253, URL Mapping, submitted 26 Feb, 1998, by General Instrument, is a step in this direction, but does not quite solve the problem.