Open Geospatial Consortium Inc.

Date: 2010-02-18

Reference number of this OGC™ document: OGC 10-034r2

Version: 0.5

Category: Candidate OGC™ Discussion Paper

Editor:   George Percivall and David Burggraf

OGC MovingObjectSnapshot:  An application schema of the OGC Geography Markup Language

Copyright notice

Copyright 2010 Open geospatial Consortium

To obtain additional rights of use, visit http://www.opengeospatial.org/legal/.

Warning

This document is not an OGC Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an OGC Standard.

Recipients of this document are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Document type:	OGC™ Discussion Paper

Document subtype:	GML Application Schema

Document stage:	Proposed

Document language:	English

Contents	Page

1	Scope	1

2	Conformance	1

3	Normative references	1

4	Conventions	2

5	Motion described by MovingObjectSnapshot	3

5.1	Intended Use Cases	3

5.2	Present Position	3

5.3	Local Tangent Plane	3

5.4	Velocity and Speed	4

6	Example of MovingObjectSnapshot	4

7	Schema for MovingObjectSnapshot	5

- Preface

This document defines the OGC MovingObjectSnapshot that encodes properties that describe the motion of an object.  The MovingObjectSnapshot is an application schema of the OGC Geography Markup Language (GML), version 3.2.   The motion described by MovingObjectSnapshot is intended for simple use cases such as a car driving through city streets or a person walking in a park.

- Document terms and definitions

This document uses the specification terms defined in Subclause 5.3 of [OGC 05-008]. In particular, the word “shall” (not “must”) is the verb form used to indicate a requirement to be strictly followed to conform to this specification

- Document contributor contact points

All questions regarding this document should be directed to the editor or the contributors:

| Name | Organization |
| --- | --- |
| George Percivall | Open Geospatial Consortium |
| Ron Lake | Galdos Systems Inc. |
| Carl Reed, PhD | Open Geospatial Consortium |
| David Burggraf, PhD | Galdos Systems Inc. |

- Revision history

| Date | Release | Editor | Primary clauses modified | Description |
| --- | --- | --- | --- | --- |
| 2010/02/13 | 0.1 | Percivall | NA | First version |
| 2010/02/18 | 0.2 | Burggraf | All | Replaced schema definition and instance. Clarified text and terminology throughout. |
| 2010/02/18 | 0.3 | Percivall | Edits throughout | Clarifications about LTP. Refined text and terminology throughout. |
| 2010/03/02 | 0.4 | Percivall | Edits throughout | Further clarification of the LTP and descriptions of the terms of motion. |
| 2010/03/02 | 0.5 | Burggraf | Edits throughout | Comments and changes throughout |

- Changes to the OGC Abstract Specification

The OGC™ Abstract Specification does not require changes to accommodate the technical contents of this document.

Foreword

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. The Open Geospatial Consortium Inc. shall not be held responsible for identifying any or all such patent rights.

Recipients of this document are requested to submit, with their comments, notification of any relevant patent claims or other intellectual property rights of which they may be aware that might be infringed by any implementation of the standard set forth in this document, and to provide supporting documentation.

OGC MovingObjectSnapshot:  An application schema of the OGC Geography Markup Language

# Scope

This document defines a GML application schema to encode a snapshot of a moving object including its location, translational velocity and acceleration. This specification addresses the use case of a solid object, such as a car, travelling in a plane local to the object, such as a street.  The velocity is an instantaneous vector composed of a scalar speed and a heading relative to North.

# Conformance

Data using this specification shall produce an XML document compliant with MovingObjectSnapshotType as defined in the schema with namespace:

http://www.opengis.net/movingObjectSnapshot

These XML Schema files are also bundled with the present document. In the event of a discrepancy between the bundled and online versions of the XML Schema files, the online files shall be considered authoritative.

# Normative references

The following normative documents contain provisions that, through reference in this text, constitute provisions of this document. For dated references, subsequent amendments to, or revisions of, any of these publications do not apply. For undated references, the latest edition of the normative document referred to applies.

OpenGIS Geography Markup Language (GML) Encoding Standard, Version 3.2.1, OGC Document 07-036

GML3.2 Simple Point Profile, Version 0.2, OGC Document 10-071r1

# Conventions

The abbreviated terms used in this document include:

CRS	Coordinate Reference System

GML	Geography Markup Language

LTP	Local Tangent Plane

OGC	Open Geospatial Consortium

For the purposes of this specification, the following terms and definitions apply.

velocity

instantaneous rate of change of position.

Note	 Velocity is a vector quantity which in this specification is composed of speed and heading.

speed

magnitude of instantaneous velocity (a scalar quantity) measured in linear units per unit time (e.g. meters per second).

heading

angle between North and the instantaneous direction of motion in the local tangent plane. The angle is measured clockwise from North.

Note: Defining clockwise direction for heading is consistent with applying the right hand rule in the Local north, east, down (NED) coordinates of the LTP.

Note: This definition assumes that the object is not located at either of the geographic poles.

local tangent plane

Euclidean plane, tangent to the Earth’s surface with origin at the position of the moving object and axis directions: Northward, Eastward and Downward, respectively.

acceleration

instantaneous rate of change of velocity.

# Motion described by MovingObjectSnapshot

## Intended Use Cases

Typical use cases for the moving object defined in this specification are a car driving through city streets or a human walking in a park.

The velocity defined in this specification is applicable to the instantaneous, translational motion of an object moving relative to a two-dimensional plane. The object may be a single point or the centre of mass of a rigid body, such as a car.  The coordinate system is a Local Tangent Plane that includes North as a coordinate axis.

The OGC Sensor Web Enablement (SWE) standards address motions and coordinate reference frames more complex that the use case of this MovingObjectSnapshot specification.

## Present Position

The instantaneous position of the object in MovingObjectSnapshot is defined using the GML Point Profile.  GML Point Profile defines the location of a point using geographic latitude and longitude in degrees. The coordinates in the GML Point Profile are defined in a coordinate reference system known as EPSG:4326 and formally identified in OGC by the URN identifier: urn:ogc:def:crs:EPSG::4326.

EPSG:4326 is a common coordinate reference system that refers to WGS84 as (latitude, longitude) pair coordinates in degrees with Greenwich as the central meridian. The exact meaning can be obtained by looking up the URN identifier (urn:ogc:def:crs:EPSG::4326) in the OGC Naming Authority register (http://www.opengis.net/register/ogc-na/name/) under the ‘NAME’ column - the definition is encoded as GML can be resolved by following the link provided in the ‘LOCATOR’ column.

Elevation is defined separate from the point location in latitude-latitude.  The vertical datum in MovingObjectSnapshot is the WGS84 EGM96 Geoid, So elevation is measured in meters above the geoid (sea level) in the direction opposite the plumb line.

## Local Tangent Plane

The velocity in MovingObjectSnapshot is defined relative to Euclidean plane corresponding to a Local Tangent Plane (LTP) at the location of the object.  The LTP is defined with its origin at the point where the object is located on (or possibly near) the Earth’s surface. LTP is oriented in three dimensions with the vertical axis taken to be straight down, parallel to the gravitational gradient, with the plumb line. The other two axes are perpendicular to the vertical axis aligned with local geographic north and east.

## Velocity and Speed

In physics, velocity is the rate of change of position. It is a vector physical quantity; both speed and direction are required to define it. In the SI (metric) system, it is measured in meters per second: (m/s) or ms−1. The scalar absolute value (magnitude) of velocity is speed. For example, "5 meters per second" is a scalar and not a vector, whereas "5 meters per second east" is a vector. The average velocity v of an object moving through a displacement (Δx) during a time interval (Δt) is described by the formula:

The rate of change of velocity is acceleration – how an object's speed or direction changes over time, and how it is changing at a particular point in time.

# Example of MovingObjectSnapshot

This example captures the motion of a car travelling at about 100 km per hour (30 meters per second) on a highway outside of Krakow Poland.

Example use of MovingObjectSnapshot.

<mos:MovingObjectSnapshot gml:id="M1">
      <mos:timeStamp>
        <gml:TimeInstant gml:id="T1">
          <gml:timePosition>2010-03-04T15:34:00Z</gml:timePosition>
        </gml:TimeInstant>
      </mos:timeStamp>
      <mos:position>
        <gml:Point gml:id="p12">
          <gml:pos> 50.0016, 20.001 </gml:pos>
        </gml:Point>
      </mos:position>
      <mos:speed uom="m/s">30</mos:speed>
      <mos:heading uom="deg">295</mos:heading>
    </mos:MovingObjectSnapshot>

NOTE: It is recommended that the uom value be the case-sensitive symbol for a unit of measure as specified in the “Unified Code of Units of Measure" (UCUM) (http://aurora.regenstrief.org/UCUM). This provides a set of symbols and a grammar for constructing identifiers for units of measure that are unique, and may be easily entered with a keyboard supporting the limited character set known as 7-bit ASCII. ISO 2955 formerly provided a specification with this scope, but was withdrawn in 2001. UCUM largely follows ISO 2955 with modifications to remove ambiguities and other problems.

# Schema for MovingObjectSnapshot

A MovingObjectSnapshot element allows the user to describe the present time, position, along with the speed, heading, acceleration and elevation of an object constrained to a plane.  The latter are not part of the object but can be added as additional properties which are defined in the schema.

The MovingObjectSnapshot is defined by the following XML Schema fragment:

<element name="MovingObjectSnapshot" type="mos:MovingObjectSnapshotType"
    substitutionGroup="gml:AbstractFeature"/>
  <!-- ================================================================== -->
  <complexType name="MovingObjectSnapshotType">
    <annotation>
      <documentation xml:lang="en">This type encapsulates the snapshot of a moving point object or
        the position/velocity/acceleration of a rigid body. The supporting use case is a moving
        vehicle or person on a local tangent plane</documentation>
    </annotation>
    <complexContent>
      <extension base="gml:AbstractFeatureType">
        <sequence>
          <element name="timeStamp" type="gml:TimeInstantPropertyType" minOccurs="0">
            <annotation>
              <documentation>value is a GML point.</documentation>
            </annotation>
          </element>
          <element name="position" type="gml:PointPropertyType">
            <annotation>
              <documentation>value is a GML point.</documentation>
            </annotation>
          </element>
          <element name="speed" type="gml:SpeedType" minOccurs="0">
            <annotation>
              <documentation>value must be in meters per second</documentation>
            </annotation>
          </element>
          <element name="heading" type="gml:AngleType" minOccurs="0">
            <annotation>
              <documentation>value must be in degrees measured clockwise from due North (due
                North = 0 degrees).</documentation>
            </annotation>
          </element>
          <element name="scalarAcceleration" type="gml:MeasureType" minOccurs="0">
            <annotation>
              <documentation>value must be in units appropriate for acceleration (e.g. meters per
                second per second)</documentation>
            </annotation>
          </element>
          <element name="elevation" type="gml:LengthType" minOccurs="0"/>
        </sequence>
      </extension>
    </complexContent>
  </complexType>